
SUMMER TRAVEL ITEMS WE LOOK FOR...
Most of these checks are included in our 5000 mile “Major” Oil & Inspection Service (100+ items)
· Proper operation of all Lights, Oil pressure/Brake fluid warning, Backup & Brake; Ignition switch binding & Driver’s Seat Belt Latch. Pitted Headlights & Windshields reduce night vision.  Dim Tail lights needing cleaning.  Washer jets & wipers are among your most important safety equipment. 
· Test for Engine Sludge buildup in ’94& later 9000, GM 900’s,  9-3’s & 9-5’s, every 5000 miles. (Treatment add’l).
· Review of Service Records for “Due” items such as Fuel, Air and Cabin Filters, Serpentine systems, Spark Plugs, Ignition Rotor, Coolant & Thermostat, Brake & Clutch Fluid, Gear box and Power steering Oils, Suspension lubrications & more.

· Tears in Axle (CV) & Steering Boots.  Spare Tire inflation (most we find are flat) 
· Fractured Belts - Swollen Coolant & Heater Hoses – Water pump leakage or rolling noises pointing to failure.

· Automatic Transmission fluid must be kept fresh and clear.  Oxidized Fluid will damage the transmission.

· We Monitor Engine bearings and adjust Oil Viscosity to match.

· Crankcase Ventilation System service is essential to long-term engine life and emissions.                                                    We FLOW TEST the Crankcase vent system to prevent Sludging.
· Clutch Release Cable adjustment.  Manual Transmission Fluid level checked if leakage is apparent (add’l chg.) 
· Overheated Cables, Voltage Regulator age, Alternator ground cable, loose Clutch Hose, Wiper trans. linkage.

· Ball Joints or Track Rod End Freeplay at wheels - Exhaust system – Oil & Fluid Leakage from critical parts.
· Fractures in lower suspension “A-Arms” on Classic 900’s

· Trans & Power Steering fluid leakages are sometimes inexpensively addressed with additives.

· Motor mounts and Torque Strut bushings.  Shift Couplers for failing.  Restricted Clutch hydraulic hoses.
· A Broken transmission mount will allow engine to shift on GM900’s & 9-3’s, damaging alternator pulley, leading to Serpentine Belt failure.
· Serpentine Belt systems should be removed & Inspected every year.  Belts older than 45,000 and pulleys older than   30,000 miles are suspect.  Failure will immobilize vehicle, disable Power Steering & overheat engine.  

· Serpentine Tensioner & Idler pulleys are more prone to failure than the belts.  Broken belts or pulleys can damage other components.  Engine overheating can cause catastrophic damage within minutes. 
· Stroboscopic and Stethoscope checking of the Serpentine Belt and accessible Pulleys for the best possible inspection. 

· Camchain and Balance Chain noises, indicating potential wear or failure.
· Radiator Inlet prone to break; Upper Cooling & Heater Hoses swell and burst.  We inspect & treat Headlight & Cooling fan relay wiring on 9000’s to prevent electrical overload and wiring damage.  

· AC Performance Tests & Compressor noises: ( Compressor failures may disable the Serpentine & Cooling systems. )
· Cooling System:  Pressure Cap & Cooling Fan operation critical during summer months. 

· Direct Ignition can fail on all 2.3 Liter models.  Carrying a Spare and replacing spark plugs on time will help.  
· Fuel Pump evaluation or replacement beyond 120,000 miles.  (Some can be Scope checked (add’l chg.) 

· Full Road Testing to check for Engine, Clutch, Transmission & Brake performance; Suspension, Tracking & Steering;  Checks for engine and ancillary noises indicating potential problems.  Notations of what we have observed and evaluated, future service or repair needs, and items to watch for.   *** prices and servicing details may change without notice.  










































































































